Below is a brief summary on M3/M3i Reset# :

1. When system suspends, +3.3V will be removed (but slowly because of loadings). But, PCIRST# will be asserted as soon as +3.3V falls below the lower limit and its slop is much steeper than power rail's. Hence, M3/M3I will see the acitve PCIRST# and it can cause a false PME# generation (only if PME# generation is enabled).

2. M3/M3i will need PCIRST# whenever its power is restored (power-on / resume from suspend) as there are certain things supposed to be determined while PCIRST# is active.

3. In order to satisfy 1 & 2 above, PCIRST# has to be isolated when system suspends but has to be connected to the chip when it resumes.

The control signal, SUSPEND*, is to block the PCIRST# at any time. This means that M3/M3i will not get the active PCIRST# when systems resumes from suspend mode for M3/M3I and MDC connector. For Mini-PCI connector, the same control signal may be required.